CPE301 – SPRING 2019

# Design Assignment 4B

Student Name: Michael Johnson

Student #: 2000878717

Student Email: johnsm17@unlv.nevada.edu

Primary Github address: https://github.com/miggnuggets/submissions.git

Directory: Repository_301

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

Atmega328P

Potentiometer

UNL2003 Driver Breakout Board

Stepper Motor

Servo Motor

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1

#define F_CPU 16000000UL

#include <avr/io.h>

#include <util/delay.h>

#include <avr/interrupt.h>

#include <stdio.h>

int start_value = 1699; // Start Timer1

// Function Declarations

void read_adc(void);

void adc_init(void);

volatile unsigned int adc_value;

volatile int OVFCount; // Setup global overflow counter

volatile double	float_value;

volatile int RPMvalue;

float c = 1699; // Needed offset for Affine Transformation

char outs[20];

int main(void)

{

adc_init();             // Start ADC

_delay_ms(125);         // Wait

TCCR1B	|= (1<<WGM12)|(1<<CS11);

/*

CTC Mode with Prescalar to 8

*/

TIMSK1 |= (1<<OCIE1A);

/*

Turn on interrupt Mask for OCR1A

*/

OCR1A = start_value; // Start Timer at longest possible value, 20,000us

sei();

//Test of timer

DDRD = 0xFF; // Set PortD as an output

OVFCount = 0; // Start global overflow counter

while (1)

{

read_adc();

/*

Affine Transformation

PWM output = adc_value*(39999-1699)/(1024-0) + 1699

*/

float_value = adc_value;

RPMvalue = (float_value*37.4033) + c;

OCR1A = RPMvalue; // Load newest time measurement value

}

}

/* Timer 1 Compare Interrupt Routine */

ISR(TIMER1_COMPA_vect)

{

OVFCount++; // Increment overflow flag counter

int count; // Start local counter

count = OVFCount; // Let local counter mimic global counter variable

// Step 1

if(count == 1){

PORTD = 0xE;

}

// Step 2

if(count == 2) {

PORTD = 0xC;

}

// Step 3

if(count == 3) {

PORTD = 0xD;

}

// Step 4

if(count == 4) {

PORTD = 0x9;

}

// Step 5

if(count == 5) {

PORTD = 0xB;

}

// Step 6

if(count == 6) {

PORTD = 0x3;

}

// Step 7

if(count == 7) {

PORTD = 0x7;

}

// Step 8

if(count == 8) {

PORTD = 0x6;

}

// Reset Counter if gone through all steps

if(count == 8){

OVFCount = 0; // Restart Stepper Counter

}

}

/* ADC */

void adc_init(void)

{

/** Setup and enable ADC **/

ADMUX = (0<<REFS1)|    // Reference Selection Bits

(1<<REFS0)|    // AVcc - external cap at AREF

(0<<ADLAR)|    // ADC Left Adjust Result

(0<<MUX2)|     // Analog Channel Selection Bits

(0<<MUX1)|     // ADC0 (PC0 PIN23)

(0<<MUX0);

ADCSRA = (1<<ADEN)|    // ADC Enable

(0<<ADSC)|     // ADC Start Conversion

(0<<ADATE)|    // ADC Auto Trigger Enable

(0<<ADIF)|     // ADC Interrupt Flag

(0<<ADIE)|     // ADC Interrupt Enable

(1<<ADPS2)|    // ADC Prescaler Select Bits

(0<<ADPS1)|

(1<<ADPS0);

}

/* READ ADC PINS */

void read_adc(void)

{

unsigned char i = 4;

adc_value = 0;

while (i--)

{

ADCSRA |= (1<<ADSC);

while(ADCSRA & (1<<ADSC));

adc_value+= ADC;

_delay_ms(50);

}

adc_value = adc_value / 4;  // Average some samples

}

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 2

#define F_CPU 16000000UL

#include <avr/io.h>

#include <util/delay.h>

#include <stdio.h>

#define BAUDRATE 9600

#define BAUD_PRESCALLER (((F_CPU / (BAUDRATE * 16UL))) - 1)

void read_adc(void);        // Function Declarations

void adc_init(void);

void USART_init( unsigned int ubrr );

void USART_tx_string( char *data );

volatile unsigned int adc_value;

volatile double	float_value;

volatile int PWMvalue;

float c = 97; // Needed offset for Affine Transformation

char outs[20];

int main(void)

{

// Configure Timer 1

TCCR1A |= (1<<COM1A1)|(1<<COM1B1)|(1<<WGM11);

/*

Non inverted PWM

Prescalar = 64

WGM Mode 14 - Fast PWM

*/

TCCR1B|=(1<<WGM13)|(1<<WGM12)|(1<<CS11)|(1<<CS10);

ICR1 = 4999;

/*

PWM Frequency = 50Hz, Period = 20ms standard for servo motor operation

*/

DDRB |= (1<<PORTB1);

adc_init();             // Start the ADC

USART_init(BAUD_PRESCALLER);  // Start the USART

USART_tx_string("Connected!\r\n");    // we're alive!

_delay_ms(125);         // wait a bit

while(1)

{

read_adc();

/*

Affine Transformation

PWM output = adc_value*(535-97)/(1024-0) + 97

*/

float_value = adc_value;

PWMvalue = (float_value*0.4277343) + c;

snprintf(outs,sizeof(outs),"%3d\r\n", PWMvalue);  // print it

USART_tx_string(outs);

OCR1A = PWMvalue;

_delay_ms(125);        // Wait

}

}

/* Start ADC */

void adc_init(void)

{

/** Setup and enable ADC **/

ADMUX = (0<<REFS1)|    // Reference Selection Bits

(1<<REFS0)|    // AVcc - external cap at AREF

(0<<ADLAR)|    // ADC Left Adjust Result

(0<<MUX2)|     // Analog Channel Selection Bits

(0<<MUX1)|     // ADC0 (PC0 PIN23)

(0<<MUX0);

ADCSRA = (1<<ADEN)|    // ADC ENable

(0<<ADSC)|     // ADC Start Conversion

(0<<ADATE)|    // ADC Auto Trigger Enable

(0<<ADIF)|     // ADC Interrupt Flag

(0<<ADIE)|     // ADC Interrupt Enable

(1<<ADPS2)|    // ADC Prescaler Select Bits

(0<<ADPS1)|

(1<<ADPS0);

}

/* READ ADC PINS */

void read_adc(void)

{

unsigned char i = 4;

adc_value = 0;

while (i--)

{

ADCSRA |= (1<<ADSC);

while(ADCSRA & (1<<ADSC));

adc_value+= ADC;

_delay_ms(50);

}

adc_value = adc_value / 4;  // Average some samples

}

/* Start USART (RS-232)  */

void USART_init( unsigned int ubrr )

{

UBRR0H = (unsigned char)(ubrr>>8);

UBRR0L = (unsigned char)ubrr;

UCSR0B = (1 << TXEN0);     // Enable RX, TX & RX interrupt

UCSR0C = (3 << UCSZ00);    //asynchronous 8 N 1

}

/* SEND A STRING TO THE RS-232 */

void USART_tx_string( char *data )

{

while ((*data != '\0'))

{

while (!(UCSR0A & (1 <<UDRE0)));

UDR0 = *data;

data++;

}

}

- SCHEMATICS

Task 1:

Task 2:

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

Task 1:

Output Waveform:

Task 2:

ADC from change in position

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

Task 1:

Task 2:

- VIDEO LINKS OF EACH DEMO

https://youtu.be/LfuonSIHfHg

- GITHUB LINK OF THIS DA

https://github.com/miggnuggets/submissions.git

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Michael Johnson