CPE301 – SPRING 2019

# MIDTERM II

Student Name: Michael Johnson

Student #:2000878717

Student Email: johnsm17@unlv.nevada.edu

Primary Github address: https://github.com/miggnuggets/submissions.git

Directory: Repository_301

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

List of Components used

Atmega328P, APDS9960, and ESP-01

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

#define F_CPU 16000000UL

#include <avr/io.h>

#include <util/delay.h>

#include <stdio.h>

#include "i2c_master.h"

#include "uart.h"

#include "apds.h"

#define BAUD 9600

#define BRGVAL (F_CPU/16/BAUD) - 1

#ifndef APDS_H

#define APDS_H

#include <avr/io.h>

#include "i2c_master.h"

#include "apds.h"

#define APDS_WRITE	(0x39 << 1) | 0

#define APDS_READ	(0x39 << 1) | 1

/* Debug */

/* APDS-9960 I2C address */

#define APDS9960_I2C_ADDR       0x39

/* Error code for returned values */

#define ERROR                   0xFF

/* Acceptable device IDs */

#define APDS9960_ID_1           0xAB

#define APDS9960_ID_2           0x9C

/* Misc parameters */

#define FIFO_PAUSE_TIME         30      // Wait period (ms) between FIFO reads

/* APDS-9960 register addresses */

#define APDS9960_ENABLE         0x80

#define APDS9960_ATIME          0x81

#define APDS9960_WTIME          0x83

#define APDS9960_PERS           0x8C

#define APDS9960_CONFIG1        0x8D

#define APDS9960_PPULSE         0x8E

#define APDS9960_CONFIG2        0x90

#define APDS9960_ID             0x92

#define APDS9960_RDATAL         0x96

#define APDS9960_RDATAH         0x97

#define APDS9960_GDATAL         0x98

#define APDS9960_GDATAH         0x99

#define APDS9960_BDATAL         0x9A

#define APDS9960_BDATAH         0x9B

#define APDS9960_POFFSET_UR     0x9D

#define APDS9960_POFFSET_DL     0x9E

#define APDS9960_CONFIG3        0x9F

/* Bit fields */

#define APDS9960_PON            0b00000001

#define APDS9960_AEN            0b00000010

#define APDS9960_PEN            0b00000100

#define APDS9960_WEN            0b00001000

#define APSD9960_AIEN           0b00010000

#define APDS9960_PIEN           0b00100000

#define APDS9960_GEN            0b01000000

#define APDS9960_GVALID         0b00000001

/* On/Off definitions */

#define OFF                     0

#define ON                      1

/* Acceptable parameters for setMode */

#define POWER                   0

#define AMBIENT_LIGHT           1

/*#define PROXIMITY               2*/

#define WAIT                    3

#define AMBIENT_LIGHT_INT       4

#define ALL                     7

/* LED Drive values */

#define LED_DRIVE_100MA         0

#define LED_DRIVE_50MA          1

#define LED_DRIVE_25MA          2

#define LED_DRIVE_12_5MA        3

/* LED Boost values */

#define LED_BOOST_100           0

#define LED_BOOST_150           1

#define LED_BOOST_200           2

#define LED_BOOST_300           3

/* Default values */

#define DEFAULT_ATIME           219     // 103ms

#define DEFAULT_WTIME           246     // 27ms

#define DEFAULT_PROX_PPULSE     0x87    // 16us, 8 pulses

#define DEFAULT_POFFSET_UR      0       // 0 offset

#define DEFAULT_POFFSET_DL      0       // 0 offset

#define DEFAULT_CONFIG1         0x60    // No 12x wait (WTIME) factor

#define DEFAULT_LDRIVE          LED_DRIVE_100MA

#define DEFAULT_PGAIN           PGAIN_4X

#define DEFAULT_AGAIN           AGAIN_4X

#define DEFAULT_AILT            0xFFFF  // Force interrupt for calibration

#define DEFAULT_AIHT            0

#define DEFAULT_PERS            0x11    // 2 consecutive prox or ALS for int.

#define DEFAULT_CONFIG2         0x01    // No saturation interrupts or LED boost

#define DEFAULT_CONFIG3         0       // Enable all photodiodes, no SAI

#define DEFAULT_GLDRIVE         LED_DRIVE_100MA

#define DEFAULT_GWTIME          GWTIME_2_8MS

void apds_init();

void readColor();

#endif

FILE str_uart = FDEV_SETUP_STREAM(uart_putchar, NULL , _FDEV_SETUP_WRITE);

char results[256];

void init_UART();

int uart_putchar( char c, FILE *stream);

int main(void){

uint16_t red = 0, green = 0, blue = 0;

i2c_init();

init_UART();

stdout = &str_uart;

apds_init();

_delay_ms(2000);

printf("AT\r\n");

_delay_ms(5000);

printf("AT+CWMODE=1\r\n");

_delay_ms(5000);

printf("AT+CWJAP=\"Wifi\",\"Password\"\r\n");

while (1) {

_delay_ms(5000);

printf("AT+CIPMUX=0\r\n");

_delay_ms(5000);

printf("AT+CIPSTART=\"TCP\",\"api.thingspeak.com\",80\r\n");

_delay_ms(5000);

readColor(&red, &green, &blue);

printf("AT+CIPSEND=104\r\n");

printf("GET https://api.thingspeak.com/update?api_key=WRITEKEY&field1=%05u&field2=%05u&field3=%05u\r\n", red, green, blue);

_delay_ms(5000);

}

}

void apds_init(){

uint8_t setup;

i2c_readReg(APDS_WRITE, APDS9960_ID, &setup,1);

if(setup != APDS9960_ID_1) while(1);

setup = 1 << 1 | 1<<0 | 1<<3 | 1<<4;

i2c_writeReg(APDS_WRITE, APDS9960_ENABLE, &setup, 1);

setup = DEFAULT_ATIME;

i2c_writeReg(APDS_WRITE, APDS9960_ATIME, &setup, 1);

setup = DEFAULT_WTIME;

i2c_writeReg(APDS_WRITE, APDS9960_WTIME, &setup, 1);

setup = DEFAULT_PROX_PPULSE;

i2c_writeReg(APDS_WRITE, APDS9960_PPULSE, &setup, 1);

setup = DEFAULT_POFFSET_UR;

i2c_writeReg(APDS_WRITE, APDS9960_POFFSET_UR, &setup, 1);

setup = DEFAULT_POFFSET_DL;

i2c_writeReg(APDS_WRITE, APDS9960_POFFSET_DL, &setup, 1);

setup = DEFAULT_CONFIG1;

i2c_writeReg(APDS_WRITE, APDS9960_CONFIG1, &setup, 1);

setup = DEFAULT_PERS;

i2c_writeReg(APDS_WRITE, APDS9960_PERS, &setup, 1);

setup = DEFAULT_CONFIG2;

i2c_writeReg(APDS_WRITE, APDS9960_CONFIG2, &setup, 1);

setup = DEFAULT_CONFIG3;

i2c_writeReg(APDS_WRITE, APDS9960_CONFIG3, &setup, 1);

}

void readColor(uint16_t *red, uint16_t *green, uint16_t *blue){

uint8_t redl, redh;

uint8_t greenl, greenh;

uint8_t bluel, blueh;

i2c_readReg(APDS_WRITE, APDS9960_RDATAL, &redl, 1);

i2c_readReg(APDS_WRITE, APDS9960_RDATAH, &redh, 1);

i2c_readReg(APDS_WRITE, APDS9960_GDATAL, &greenl, 1);

i2c_readReg(APDS_WRITE, APDS9960_GDATAH, &greenh, 1);

i2c_readReg(APDS_WRITE, APDS9960_BDATAL, &bluel, 1);

i2c_readReg(APDS_WRITE, APDS9960_BDATAH, &blueh, 1);

*red = redh << 8 | redl;

*green = greenh << 8 | greenl;

*blue = blueh << 8 | bluel;

}

void init_UART(void){

//Set baud rate

uint16_t baud_rate = BRGVAL;

UBRR0H = baud_rate >> 8;

UBRR0L = baud_rate & 0xFF;

//Enable receiver and transmitter

UCSR0B = ( 1 <<RXEN0)|( 1 <<TXEN0);

// Set frame format: 8data, 1stop bit

UCSR0C = (3 <<UCSZ00);

}

int uart_putchar(char c, FILE *stream){

//wait until buffer empty

while ( !( UCSR0A & ( 1 <<UDRE0)) );

//Put data into buffer

UDR0 = c;

return 0;

}

- SCHEMATICS

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

https://youtu.be/3_gNeESOjLs

- GITHUB LINK OF THIS DA

https://github.com/miggnuggets/submissions.git

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Michael Johnson